Detection and Discrimination of Marine Oil Spills and Look-alike Phenomena in Synthetic Aperture Radar Imagery

Xudong Huang1, Biao Zhang1, William Perrie2, Jinyu Sheng1

1Department of Oceanography, Dalhousie University, Halifax, NS B3H 4R2, Canada

2Fisheries and Oceans Canada, Bedford Institute of Oceanography, Dartmouth, NS B2Y 4A2, Canada

Abstract

Marine oil spills and look-alike phenomena, such as biogenic slicks, low-wind areas, rain cells, and sea ice, are manifest as dark patches in wide-swath synthetic aperture radar (SAR) imagery, posing significant challenges for reliable detection and discrimination. To address this problem, we propose the Multi-class Dark Object Detection Network (MDODNet), a dedicated framework for simultaneous detection and classification of these dark formations. MDODNet was trained and validated on a curated dataset of 11,040 annotated dark patches extracted from 2,192 Sentinel-1 VV-polarization SAR images, with annotations cross-validated using multi-source auxiliary data. The network design enhances feature representation through deeper layers and multi-scale feature fusion, while detection robustness is achieved via an anchor-based detection head optimized with “Focal and Efficient Intersection over Union” loss. Experimental results show strong detection performance, achieving average precisions of 87.04% for biogenic slicks, 93.28% for low-wind areas, 88.82% for oil spills, 90.04% for rain cells, and 89.16% for sea ice. For oil spill cases, detected regions are further refined using the UNet++ model to precisely delineate oil spill boundaries. Case studies demonstrate the strong generalization ability of MDODNet across diverse SAR datasets, underscoring its potential as a reliable and intelligent tool for operational marine oil spill monitoring.